APPLICANT INFORMATION

This checklist is designed to assist with the documentation and information that needs to be provided with your application. The comments provided in italics are references to paragraphs of the Acceptable Solutions unless specified otherwise. These are for guidance only and may not be the specific means of compliance for your project. Please complete all orange sections (left side of form) or select N/A where relevant This checklist is suitable for new buildings, additions to existing buildings and alterations (more than minor) for the following: - Residential multi-unit buildings outside scope of C/AS1 - Communal Residential, Communal Non-Residential, Commercial, Industrial buildings

| CS19C Commercial Building Consent Application and Processing Checksheet |  |
| --- | --- |

| # PROPERTY INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| APPLICANT TO COMPLETE |  | COUNCIL USE ONLY |  |  |
|  |  | Yes | RFI | Reasons for Decisions/Comments |
| Clause A1 - Classified Uses | Choose an item. | ☐ | ☐ | BC NUMBER: Click to enter text. Check project information provided by applicant. |
| Use of Building | Choose an item. |  |  |  |
| Building Importance Level | Choose an item. |  |  |  |
| E2 Risk Matrix Score | Click to enter text. (Highest of all faces) |  |  |  |
| Earthquake Zone | Choose an item. |  |  |  |
| Wind Zone | Choose an item. |  |  |  |

| Select | Form 2: Application for Building Consent | COUNCIL USE ONLY |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| ☐ | All sections of Form 2 completed | ☐ | ☐ | ☐ |  |

| Building Act 2004 |  |  |  |  | COUNCIL USE ONLY |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Section 53 Building Levy (accurate value of work for project) |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 31 – 39 Project Information Memoranda (PIM/PIC) (review/note Liquefaction category, minimum floor heights achieved if applicable, contamination, services) |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 71 – 74 Natural Hazards associated with this consent (see PIM information). Applicant to select as applicable: ☐Erosion (i.e. bank erosion and sheet erosion) ☐Falling debris (i.e. rock, snow and ice) ☐Subsidence ☐Inundation (i.e. flooding, surge and ponding) ☐Slippage Complete CS 12.1 Natural Hazards Checksheet |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 75 – 83 Construction of building on 2 or more allotments Complete AF 13.1 Section 75 Certificate Building over 2 or more allotments |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 112 Alterations to existing buildings |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 115 Change of use |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 113 Buildings with specified intended lives |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 67 Waivers or Modifications Complete AF WM Application for Waiver or Modification |  |  |  |  | ☐ | ☐ | ☐ |  |
| Section 26 Warnings or Bans |  |  |  |  | ☐ | ☐ | ☐ |  |
| Yes | N/A |  | Producer Statements |  | COUNCIL USE ONLY |  |  |  |
|  |  |  |  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| ☐ | ☐ |  | Producer Statements Check: - Building address - scope of work/ building code clauses covered - drawings referenced - verification of design assumptions - construction monitoring and inspection schedule - authors qualification and registration (check on EngNZ register) - signed and dated - supporting documentation (calculations/test report) |  | ☐ | ☐ | ☐ |  |
| PS2 Requirements: In some higher risk applications, the BCA may require the design to be validated by a PS2 for some structure, fire and façade design. Further information can be found here |  |  |  |  |  |  |  |  |
| Yes |  | N/A |  | Other Applications | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  |  |  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| ☐ |  | ☐ |  | Connection to Council services application provided/required (for new drainage connections or driveways) ☐ Connecting to Council Services Form | ☐ | ☐ | ☐ |  |
| ☐ |  | ☐ |  | Building over or near a council pipe or drain: within council easement or within influence zone of council pipe/drain, engineering approval provided/required ☐ Building Over Services Application form | ☐ | ☐ | ☐ |  |
| Yes |  | N/A |  | Site Plan | COUNCIL USE ONLY |  |  |  |
|  |  |  |  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| ☐ |  | ☐ |  | - Siting dimensions on site plan (minimum 3 dimensions) - Boundaries align with Title plan - Datum/contours provided | ☐ | ☐ | ☐ |  |

| Alterations to an existing building(s) or Change of Use |  |
| --- | --- |
| ☐ N/A (Skip to next section - B Stability) ☐ Alteration to an existing building? Yes (does not apply to first fit outs as this is new work and will be assessed under Section 17) |  |
| Section 112 applies – Means of escape from fire and Access and Facilities for persons with disabilities ☐ Is a change of use proposed? Yes (see Building Regulations 2005 to establish if change of use proposed) Section 115 applies – Means of escape from fire, protection of other property, sanitary facilities, structural performance and fire-rating performance |  |
| Current use: Choose an item. | Proposed use: Choose an item. |
| If yes to either of the above, complete table below to establish the Building Score and provide applicable level of assessment for the whole building. For unit titles or alterations to tenancies within an existing building, the whole building must be assessed. MBIE Building Score Methodology used to establish the level of information that needs to be provided to review relevant Code Clause requirements |  |

| BUILDING SCORE |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type |  | Key Factors |  |  |  |  | Points | Score (select as applicable) |
| Likelihood of existing building complying |  | Building Age |  |  |  |  |  |  |
|  |  | Approved from 1 June 2001 onwards |  |  |  |  | 0 | ☐ |
|  |  | Approved between 1 January 1993 and 31 May 2001 |  |  |  |  | 1 | ☐ |
|  |  | Approved on or before 31 December 1992 |  |  |  |  | 3 | ☐ |
|  |  | Information held on the building by the BCA or TA (score one of these only and choose the most comprehensive assessment) |  |  |  |  |  |  |
|  |  | For buildings approved from 1 June 2001: no consents made |  |  |  |  | 0 | ☐ |
|  |  | Full building assessment on file dated 1 June 2001 or later |  |  |  |  | 2 | ☐ |
|  |  | Full building assessment on file dated on or before 31 May 2001 |  |  |  |  | 4 | ☐ |
|  |  | One or more partial building assessments on file |  |  |  |  | 6 | ☐ |
|  |  | No assessment on file for building additions or alterations |  |  |  |  | 8 | ☐ |
|  |  | Unable to determine history of building |  |  |  |  | 8 | ☐ |
| Extent of proposed work |  | Extent of the proposed work |  |  |  |  |  |  |
|  |  | Minor |  |  |  |  | 0 | ☐ |
|  |  | Moderate |  |  |  |  | 3 | ☐ |
|  |  | Significant |  |  |  |  | 6 | ☐ |
| Potential consequences of not complying |  | Building Importance Level |  |  |  |  |  |  |
|  |  | Level 1 |  |  |  |  | 0 | ☐ |
|  |  | Level 2 |  |  |  |  | 4 | ☐ |
|  |  | Level 3 |  |  |  |  | 8 | ☐ |
|  |  | Level 4 and Level 5 |  |  |  |  | 12 | ☐ |
|  |  | Additional points for building level 1, 2 or 3 with sleeping facilities |  |  |  |  | 4 | ☐ |
| TOTAL SCORE |  |  |  |  |  |  |  | Choose an item. |
| # Assessments to be provided in relation to Building Score |  |  |  |  |  |  |  |  |
| 1-11 | List of current features plus Statement of Change |  |  |  |  |  |  |  |
| 12-19 | Gap Assessment |  |  |  |  |  |  |  |
| 20+ | Full Assessment |  |  |  |  |  |  |  |
| Further guidance on levels of assessment can be found on the MBIE website: Recommended approach |  |  |  |  |  |  |  |  |
| Alterations or Change of Use to existing building |  |  | Yes | N/A | RFI | COUNCIL USE ONLY |  |  |
| Building Score sheet provided and agree with assessment level |  |  | ☐ | ☐ | ☐ |  |  |  |

| # B STABILITY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ N/A | B1 Structure: Landscape Retaining Walls | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Construction details provided: footings, wall construction, height, surcharge correctly factored into design Excavations potentially affecting neighbouring properties mitigated and all work (including drainage) within property boundary | ☐ | ☐ | ☐ |  |
|  | Subsoil drainage, silt trap or similar prior to connection to stormwater | ☐ | ☐ | ☐ |  |
|  | Barrier specified and details provided | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Strip Footings | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Footing details depth, size, steps due to ground slope, MPa, reinforcing (size, type, centres, grade), details provided for steps in foundations (due to ground slope) | ☐ | ☐ | ☐ |  |
|  | Columns and posts pads (size, type, centres, grade) | ☐ | ☐ | ☐ |  |
|  | Underpinning/screw piles | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Slabs | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | ☐ Slab-on-grade ☐ Raft ☐ Suspended Floor Check suitability of foundation in relation to PIM/PIC, Liquefaction category or geotechnical report | ☐ | ☐ | ☐ |  |
|  | Foundation plan/slab details: - Foundation design conforms to site geotechnical parameters (if applicable) - Slab thickness, reinforcing cover. Mesh type and size - Shrinkage control joints, supplementary reinforcing bars, free joints - Base preparation: sand, hard-fill - Damp proof membrane (DPM) | ☐ | ☐ | ☐ |  |
|  | Foundation/footing details: - Foundation edge/footing detail(s) with dimensions, reinforcing size, grade and concrete strength - Point loads pads / slab thickenings shown on foundation plans - Additions to existing slab: connection to existing slab (joint preparation & starters) - Column/post: foundations/footings | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Foundation Wall – Concrete/Masonry | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Construction details: footing details (including steps in foundation), wall height, (maximum), vertical and horizontal reinforcing / starters, size and centres, masonry type and grout strength (MPa), subfloor ventilation. | ☐ | ☐ | ☐ |  |
|  | Retaining structures: subsoil drainage details silt trap or similar prior to connection to stormwater, tanking and protection specified (check tanking system details), strapped/lining details | ☐ | ☐ | ☐ |  |
|  | Seacant piles, soldier piles or precast piles | ☐ | ☐ | ☐ |  |

| ☐ N/A | B1 Structure: Subfloor Foundation Framing and Bracing | ☐ N/A |  |  | COUNCIL USE ONLY |
| --- | --- | --- | --- | --- | --- |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Pile foundation plan: type specified and details provided (ordinary, driven (SED), anchor, braced), footing details, depth, size, centres, treatment, heights, point loads | ☐ | ☐ | ☐ |  |
|  | Subfloor bracing details: type, location, length, 5.0m centres max, 4 bracing elements minimum, evenly distributed Subfloor bracing calculations: zone, demand, capacity Diaphragms >100 BUs, location, limitations 2-storey (height versus width) | ☐ | ☐ | ☐ |  |
|  | Bearers: size, centres, treatment, span, point loads, cantilever, loading, fixing to foundation walls | ☐ | ☐ | ☐ |  |
|  | Joists: size, centres, span, point loads, cantilever joists (check load bearing walls), loading, penetrations Lateral support mid-span, blocking, bracing lines | ☐ | ☐ | ☐ |  |
|  | Flooring thickness, clearances, type | ☐ | ☐ | ☐ |  |
|  | Base cladding: type, support, access, ventilation, crawlspace (450 mm minimum), vermin proofing, obstructions, large area, limited cross flow | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Mid-floor | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Mid floor structure: timber floor joist, suspended concrete slab, support beams/posts, SED etc. Floor joists details: size, centres, span, treatment, floor beams, cantilever, floor load, point loads Flooring type and diaphragm (if applicable) | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: SED Structural Steel Frame and Concrete Structures | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | SED: PS1, PS2, proprietary systems PS2 provided where: - Retaining structures over 5 m in height - New buildings and significant alterations to buildings with Importance Levels 3, 4 and 5 - New buildings greater than 5 levels above ground - Structures containing structural member spans greater than 20 m - Untested alternative solutions for structural designers - Other buildings the BCA deems of a complex nature | ☐ | ☐ | ☐ |  |
|  | Structural documentation: all structures requiring SED, covered by design (Design features report), all referenced details and sections provided, adequate dimensioning provided | ☐ | ☐ | ☐ |  |
|  | Demolition – site monitoring of adjacent buildings, partial removal of structural elements/temporary support | ☐ | ☐ | ☐ |  |
|  | Seismic strengthening included in consent application | ☐ | ☐ | ☐ |  |
|  | Concrete precast panel details and fixings provided, concrete strengths and reinforcing cover detailed on plans | ☐ | ☐ | ☐ |  |
|  | Portal frames, structural steel frames, space frames, girts, etc. | ☐ | ☐ | ☐ |  |
|  | Miscellaneous: Racking systems, ground anchors etc. | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Decks Construction (ground floor or external decks above ground floor) | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Deck piles: layout, type, size, footing depth, centres, treatment, height, connections Subfloor deck bracing (if projects >2.0m from the building) | ☐ | ☐ | ☐ |  |
|  | Bearers: size, span, treatment | ☐ | ☐ | ☐ |  |
|  | Joists: 2.0 kPa, size, span, centres, cantilever, saddle flashings | ☐ | ☐ | ☐ |  |
|  | Decking: material, type, fixings, treatment, finished floor level | ☐ | ☐ | ☐ |  |
|  | Stringer: stair construction / deck stringer, size, span, fixings, air-gap | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Walls (all levels) | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Framing (NZS3604, SED, Steel): treatment, grade, size | ☐ | ☐ | ☐ |  |
|  | Bottom and top plate: size, point loads, bracing | ☐ | ☐ | ☐ |  |
|  | Stud / post: size, height, spacing, fixings, uplift | ☐ | ☐ | ☐ |  |
|  | Structure / load path: beams / posts, SED, timber, steel, flitch, LVL, fixing details provided | ☐ | ☐ | ☐ |  |
|  | Lintel: size, point loads, fixings | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Wall Bracing (all floors) | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Wall bracing calculations - zone, demand, capacity | ☐ | ☐ | ☐ |  |
|  | Wall bracing plan - location of bracing elements, 6.0m centres, evenly distributed, wings, blocks, wet areas, split or discontinuous levels, type, location, length Dragon ties to extend brace lines to 7.5m > 100 Bus, location Ceiling diaphragm - >100 BUs, location, limitations, construction details | ☐ | ☐ | ☐ |  |
| ☐ N/A | B1 Structure: Roof and Ceiling Framing | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Trusses - Design Statement, software statement, truss layout, pitch specified | ☐ | ☐ | ☐ |  |
|  | Rafter / ridge beam / beams / roof purlins Size, span, support, fixings, treatment | ☐ | ☐ | ☐ |  |
|  | Roof bracing | ☐ | ☐ | ☐ |  |
|  | Purlin, tile batten, size, span, centres, fixings, treatment | ☐ | ☐ | ☐ |  |
|  | Ceiling /Rondo battens Size, span, centres, fixings, ceiling lining | ☐ | ☐ | ☐ |  |
|  | Suspended ceilings (seismic restraint and PS1) and ceiling lining - type, thickness, fixing | ☐ | ☐ | ☐ |  |
| B2 Durability |  |  |  |  |  |
| B2 Durability must always be considered when demonstrating compliance with each of the clauses of the Building Code. In other words, it ensures that a building will continue to satisfy the performance of the Building Code throughout its specified intended life. Under the clause, building materials, components and construction methods are required to be sufficiently durable. They must ensure that the building, without reconstruction or major renovation, continues to satisfy the other functional requirements of the Building Code throughout its life. B2 specifies minimum durability periods building elements must meet with only normal maintenance, being not less than 50, 15 or 5 years. |  |  |  |  |  |

| # C1- C6 PROTECTION FROM FIRE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Part 1: General |  | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided | ☐ | ☐ | ☐ |  |
|  | Scope of risk group and Primary risk group (if more than one) | ☐ | ☐ | ☐ |  |
|  | Occupant densities and occupant load | ☐ | ☐ | ☐ |  |
|  | Fire Engineering Brief (FEB) provided where C/VM2 or Alternative Solution used for means of compliance with stakeholder sign off | ☐ | ☐ | ☐ |  |
|  | PS2 provided when: - C/VM2 is used as means of compliance or; - Alternative solutions provided to buildings listed in Section 75(1) Fire and Emergency New Zealand Act | ☐ | ☐ | ☐ |  |
| Part 2: Firecells, Fire Safety Systems & FRR |  | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Firecell floor area limit | ☐ | ☐ | ☐ |  |
|  | Fire safety systems Type 9, 15, 18 system required? Manual and automatic fire alarm systems – type specified (Type 2, 3, 4, 5, 6, 7). Location shown on plan (heat/smoke detectors, manual call points, fire alarm panel, hold open device for doors, smoke extract) | ☐ | ☐ | ☐ |  |
|  | Is direct connection between the alarm system and FENZ needed and specified? | ☐ | ☐ | ☐ |  |
|  | Fire resistance ratings (life rating/property rating) | ☐ | ☐ | ☐ |  |
| Part 3: Means of escape |  | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Escape routes: number, height and width, length | ☐ | ☐ | ☐ |  |
|  | Escape from basements | ☐ | ☐ | ☐ |  |
|  | Open paths / Special cases of open paths | ☐ | ☐ | ☐ |  |
|  | Dead ends | ☐ | ☐ | ☐ |  |
|  | Exitways / Control of exitway activities | ☐ | ☐ | ☐ |  |
|  | External escape routes | ☐ | ☐ | ☐ |  |
|  | Final exits (CA, WB, VP) | ☐ | ☐ | ☐ |  |
|  | Single escape routes | ☐ | ☐ | ☐ |  |
|  | Special conditions for safe paths (SM) | ☐ | ☐ | ☐ |  |
|  | Doors subdividing escape routes (door features, fixings, direction, vision panel, hold open devices) Panic fastenings (CA) | ☐ | ☐ | ☐ |  |
| Part 4: Control of Internal Fire & Smoke Spread |  | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Firecells | ☐ | ☐ | ☐ |  |
|  | Glazing in fire and smoke separations | ☐ | ☐ | ☐ |  |
|  | Structural stability during fire (Structural engineer) | ☐ | ☐ | ☐ |  |
|  | Fire stopping / firecell construction | ☐ | ☐ | ☐ |  |
|  | Special requirements for sleeping areas (SM, SI) | ☐ | ☐ | ☐ |  |
|  | Specific requirements for theatres, exhibitions and retail areas (CA, WB, WS) Tiered seating (CA) | ☐ | ☐ | ☐ |  |
|  | Exitways (CA, WB, WS, VP) | ☐ | ☐ | ☐ |  |
|  | Intermittent activities | ☐ | ☐ | ☐ |  |
|  | Protected shafts | ☐ | ☐ | ☐ |  |
|  | Long corridor subdivision | ☐ | ☐ | ☐ |  |
|  | Floors: intermediate, subfloor spaces and concealed spaces (floors/walls/roof/ceiling space) | ☐ | ☐ | ☐ |  |
|  | Closures in fire and smoke separations | ☐ | ☐ | ☐ |  |
|  | Interior surface finishes, floor covering and suspended flexible fabrics | ☐ | ☐ | ☐ |  |
|  | Building services plant (Type 9) | ☐ | ☐ | ☐ |  |
| ☐ N/A | Part 5: External Fire Spread | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Relevant boundary (actual or notional) distance measured from cladding not foundation | ☐ | ☐ | ☐ |  |
|  | Fire separation for building with more than on title (subdivided) | ☐ | ☐ | ☐ |  |
|  | External walls: Horizontal fire spread, fire resistance rating, small openings and fire resisting glazing and table method for external walls | ☐ | ☐ | ☐ |  |
|  | Horizontal fire spread from roofs and open sided buildings | ☐ | ☐ | ☐ |  |
|  | Vertical fire spread | ☐ | ☐ | ☐ |  |
|  | Exterior cladding systems | ☐ | ☐ | ☐ |  |
| ☐ N/A | Part 6: Firefighting | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Fire and Emergency vehicular access | ☐ | ☐ | ☐ |  |
|  | Information for firefighters Control panel location approval from FENZ provided with application (only required where multiple zones and multiple entrances) | ☐ | ☐ | ☐ |  |
|  | Firefighting facilities | ☐ | ☐ | ☐ |  |
| ☐ N/A | Part 7: Prevention of Fire Occurring | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Fireplaces: Solid fuel, Gas burner, Oil fire, Open fire | ☐ | ☐ | ☐ |  |

| # D ACCESS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ N/A | D1 Access Routes (Internal and External) | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Accessible route(s): principal entrance, location, slope, change in levels, height clearances, obstructions | ☐ | ☐ | ☐ |  |
|  | Access Routes: slip resistance, width and protection from falling | ☐ | ☐ | ☐ |  |
|  | Lift required? (D1.3.4(c) or NZS4121) Occupancy based on para. 1.4 or para. 3.1 C/VM2 (Also determinations: 1995/008; 2003/05; 2005/49; 2006/73, 2017/084) | ☐ | ☐ | ☐ |  |
|  | Ramps: slope, slip resistance, width, landings, kerb ramps, handrails details | ☐ | ☐ | ☐ |  |
|  | Stairways: Pitch, risers and treads, width, landings, curved and spiral stairways, stair winders and visibility of stair treads, handrail details | ☐ | ☐ | ☐ |  |
|  | Fixed ladders: general, step-type ladders, rung-type ladders and individual run-type ladders | ☐ | ☐ | ☐ |  |
|  | Door and Openings | ☐ | ☐ | ☐ |  |
|  | Places of assembly | ☐ | ☐ | ☐ |  |
|  | Accessible accommodation units of communal residential buildings: number of units and facilities to be provided | ☐ | ☐ | ☐ |  |
|  | Movement of vehicles carparking areas and circulation routes (including assessment of accessible carparks) | ☐ | ☐ | ☐ |  |
|  | Other Acceptable Solutions: AS1657 - stairs, ladders, platforms and walkways for services and maintenance personnel | ☐ | ☐ | ☐ |  |
| ☐ N/A | D2 Mechanical Installations for access | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions / Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Design calculations, specifications, plans, PS1/CodeMark certificate provided D2/AS1 (NZS4332 or EN81-20), D2/AS2 (NZS4334), D2/AS3 (EN115) Accessible features detailed for lift: controls panel, lift indicators, car size, door size, handrails and space in front. NZS4212 (Section 9), Appendix C NZS4121 (Design for access and mobility – Alterations to Buildings) or NZS4334 | ☐ | ☐ | ☐ |  |

| # E MOISTURE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ N/A | E1 Surface Water | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Stormwater type: Council reticulation, private reticulation, soak pit etc. Stormwater layout plan: size and gradient, access (rodding/inspection points) Access for maintenance (rodding points/inspection points etc.) Strip drain/channel drain: connection into stormwater with silt trap or similar prior | ☐ | ☐ | ☐ |  |
|  | Stormwater run-off (sumps, catchment area, soakage, pump, pump capacity) Finished floor level of suspended floors or slabs checked in relation to surrounding land and road crown | ☐ | ☐ | ☐ |  |
|  | Downpipes, external & internal gutter size, distribution, spreaders (to lower roof) | ☐ | ☐ | ☐ |  |
| ☐ N/A | E2 External Moisture: Roof Cladding | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Roof cladding type(s), roof pitch, compatibility of materials (refer E2/AS1 Table 7 and para. 8.2 – 8.5) | ☐ | ☐ | ☐ |  |
|  | Metal roof flashings: change in pitch, roof/wall ridge, barge, fascia, apron flashings (transverse & parallel), gutter/wall junction, valley, hidden and external gutters | ☐ | ☐ | ☐ |  |
|  | Metal roof penetrations: small pipes (terminal vent/roof ventilation), soaker flashings (flue or similar), skylights, chimneys (cricket/apron flashings) | ☐ | ☐ | ☐ |  |
|  | Membrane roof: thickness, substrate, support, rainwater head/scupper/overflow, details: eaves/verge, internal gutters, apron, penetrations/ventilation Membrane deck: thickness, fall, substrate, support, drainage, junction to wall, verge/eave and rainwater head/scupper/overflow, penetrations/ventilation | ☐ | ☐ | ☐ |  |
|  | Parapets / enclosed barriers: slope, framing, capping material, drainage, junctions with wall or roof cladding | ☐ | ☐ | ☐ |  |
| ☐ N/A | E2 External Moisture: Wall Claddings | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Façade PS2 provided where: - E2/VM1 or E2/VM2 and; - Greater than 10 m in height and; - Contains more than 10 household/accommodation units | ☐ | ☐ | ☐ |  |
|  | Wall cladding type(s): suitable for risk score, compatibility of materials, product specifications provided, stud & dwang spacing matches cladding system requirements Drained cavity: batten size, horizontal/vertical battens, treatment, vermin proof, closed off to roof, fire rated cavity battens | ☐ | ☐ | ☐ |  |
|  | Masonry or schist slip/veneer: walls ties and lintels (schist veneer ties and lintels SED), stud & dwang spacing matches cladding system requirements (schist slip) Concrete or concrete masonry block: concrete strength/finish/coating, control joints (E2/AS3 – Concrete NZ CoP) | ☐ | ☐ | ☐ |  |
|  | Bottom of cladding: detail and clearances to ground, deck threshold, deck to cladding, enclosed balustrade cladding clearance to deck | ☐ | ☐ | ☐ |  |
|  | Wall junction details: external/internal corners, change of cladding junction, control joints, soffit to wall junction | ☐ | ☐ | ☐ |  |
|  | Window/door construction specified and detailed in window/door details (timber, aluminium, other) | ☐ | ☐ | ☐ |  |
|  | Window /door details: head, sill and jamb, rebate | ☐ | ☐ | ☐ |  |
|  | Penetrations: structures attached to dwelling (pergola), pipes/services, meterbox/gas califont and inter storey junctions | ☐ | ☐ | ☐ |  |
| ☐ N/A | E3 Internal Moisture | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Thermal resistance walls / ceiling; thermal break for steel framing Applies to Communal residential only | ☐ | ☐ | ☐ |  |
|  | Condensation control Applies to Communal residential only (single glazed windows) | ☐ | ☐ | ☐ |  |
|  | Waterproofing (type): overflow/ containment provision (other property), floor wastes | ☐ | ☐ | ☐ |  |
|  | Watersplash: Showers, urinals, baths – impervious surfaces walls and floors and junction details provided | ☐ | ☐ | ☐ |  |
|  | Vapour barrier to roof / wall cavity spaces in high humidity rooms such as indoor pools or saunas | ☐ | ☐ | ☐ |  |

| # F Safety of Users |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ N/A |  | F1 Hazardous agent on site | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Contamination PIM/PIC information reviewed. Check whether remediated through subdivision consent. | ☐ | ☐ |  | ☐ |  |  |
|  |  | Site investigation report and assessment F1/VM1 and remedial actions provided (detailed) Hazard to building elements (preventative measures detailed) | ☐ | ☐ |  | ☐ |  |  |
| ☐ N/A |  | F2 Hazardous building materials | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Glazing specifications provided for windows/doors and barriers (i.e. shown on elevations or window/door schedule provided) | ☐ | ☐ |  | ☐ |  |  |
|  |  | Manifestation to full height glazing - unimpeded path of travel | ☐ | ☐ |  | ☐ |  |  |
| Asbestos New building materials can be expected to be free of asbestos but many buildings constructed before the 1990s do include materials that contain asbestos. Therefore, for alterations to older buildings, Clause F1 ‘Hazardous Agents on Site’ may be relevant because an existing building can be considered as part of a site. The handling of asbestos-containing materials is covered by the Health and Safety at Work (Asbestos) Regulations 2016. These Regulations require the Licensing of asbestos removalists and asbestos assessors. Refer to the WorkSafe New Zealand website https://www.worksafe.govt.nz/topic-and-industry/asbestos/ |  |  |  |  |  |  |  |  |
| ☐ N/A |  | F3 Hazardous Substances and processes | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Location test certificate or exempt laboratory with PS1 provided | ☐ | ☐ |  | ☐ |  |  |
|  |  | X-ray, radioactive equipment | ☐ | ☐ |  | ☐ |  |  |
| Note: Hazardous Substances regulated and enforced by WORKSAFE |  |  |  |  |  |  |  |  |
| ☐ N/A |  | F4 Safety from falling | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Barrier located in spaces frequented by children under 6 years old or Barrier located in spaces NOT frequented by children under 6 years old | ☐ | ☐ |  | ☐ |  |  |
|  |  | Barriers to stairs, ramps and other location (height, top / bottom rails, spacing and openings) (areas) (para. 1.2 & Table 1) | ☐ | ☐ |  | ☐ |  |  |
|  |  | Fixed seating to decks (para. 1.2.4) | ☐ | ☐ |  | ☐ |  |  |
|  |  | Barriers or restrictors to window openings (para. 2.0) | ☐ | ☐ |  | ☐ |  |  |
|  |  | Roofs area with permanent access to have barriers | ☐ | ☐ |  | ☐ |  |  |
| ☐ N/A |  | F5 Construction and Demolition Hazards | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Work site barriers (site fences and hoardings) (para. 1.0 & 1.1), water hazards fences (para. 1.2) or gantries and toeboards (para. 1.4) | ☐ | ☐ |  | ☐ |  |  |
| ☐ N/A |  | F6 Visibility in Escape Routes | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Alteration to an existing building or Change of use assessment provided | ☐ | ☐ |  | ☐ |  |  |
|  |  | Emergency lighting plan provided: location, illuminance, duration and PS1 provided in relation to emergency lighting design | ☐ | ☐ |  | ☐ |  |  |
| ☐ N/A |  | F8 Signs | ☐ N/A |  |  |  |  | COUNCIL USE ONLY |
|  |  |  | Yes | N/A |  | RFI |  | Reasons for Decisions/ Comments |
| Reference pages |  | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ |  | ☐ |  |  |
|  |  | Exit and Fire Signage: Exit signs, call points, fire and smoke doors. Visible at all times - illumination (type) | ☐ | ☐ |  | ☐ |  |  |
|  |  | Accessibility Signage: for accessible route and facilities (car parks, entrance, route, services), assistive listening system | ☐ | ☐ |  | ☐ |  |  |
|  |  | Hazard Signage: hazardous substances and processes (HSNO CoP2.1 09-04) electrical, lifts, machine rooms, escalators/moving walks, water supplies | ☐ | ☐ |  | ☐ |  |  |
| ☐N/A | F9 Restricting access to residential pools / F4 Safety from falling |  | ☐ N/A |  |  |  | COUNCIL USE ONLY |  |
|  |  |  | Yes |  | N/A | RFI | Reasons for Decisions / Comments |  |
| Reference pages | Residential = hotel, motel, hostel, boarding house, nursing home/hospice, retirement village/rest home, campground or similar Pool barriers: surrounding pool, on property boundary, balconies projecting into immediate pool area, pool wall, strength (refer para. 2.0) Pool wall where fall greater than 1m – F4 to be considered (see Determinations for guidance) Gates in pool barriers (refer para. 3.0) Building wall forming the pool barrier (refer para. 4.0) |  | ☐ |  | ☐ | ☐ |  |  |

| # G SERVICES AND FACILITIES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ N/A | G1 Personal Hygiene | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Number of WC pans, basins, urinals, baths and showers (single sex/unisex) Refer to fire report occupancy number of occupancy number in relation to facilities required or Table 4. | ☐ | ☐ | ☐ |  |
|  | All bathroom facilities: location layout, space, unisex, basins, showers, line of sight, cubicles, lobbies Accessible facilities: access/location, number, layout | ☐ | ☐ | ☐ |  |
|  | Location of sanitary fixtures: space, access to food prep area work, basin communal | ☐ | ☐ | ☐ |  |
| ☐ N/A | G2 Laundering | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Laundering facilities: service connections, minimum space, number of facilities, accessible facilities (campgrounds) | ☐ | ☐ | ☐ |  |
| ☐ N/A | G3 Food preparation and prevention of contamination | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Domestic Appliances and Facilities: sink, cooker, food storage, energy source, space and wall linings (housing, work camps, old people’s homes and early childhood centres) | ☐ | ☐ | ☐ |  |
|  | Accessible preparation area provided where required (campgrounds and accessible accommodation units in communal residential buildings) | ☐ | ☐ | ☐ |  |
|  | Commercial and Industrial facilities: interior linings for food processing and other areas, wash down areas | ☐ | ☐ | ☐ |  |
| ☐ N/A | G4 Ventilation | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Natural ventilation: accommodation units and carparks (apartments, hotels, motels) | ☐ | ☐ | ☐ |  |
|  | Combined natural ventilation and mechanical ventilation (accommodation units) | ☐ | ☐ | ☐ |  |
|  | Mechanical ventilation: outdoor air supply, air-handling systems, extract ventilation etc. car park ventilation, PS1 provided for design | ☐ | ☐ | ☐ |  |
|  | Ventilation of spaces containing gas fuel appliances: natural/mechanical ventilation, flue construction | ☐ | ☐ | ☐ |  |
| ☐ N/A | G5 Interior Environment | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Accessible reception counters or desks (G5.3.4 – communal residential, communal non-residential and commercial buildings) | ☐ | ☐ | ☐ |  |
|  | Temperature control details: habitable spaces, bathrooms and recreation rooms applies to early childhood centres and old people’s homes | ☐ | ☐ | ☐ |  |
|  | Space: Living, dining and sleeping areas in old people’s home | ☐ | ☐ | ☐ |  |
|  | Listening systems and hearing loops in cinemas, theatres, public halls and communal non-residential with >250 people, assembly spaces in old people’s homes with >20 people | ☐ | ☐ | ☐ |  |
| ☐ N/A | G6 Airborne and impact sound | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | STC and IIC ratings for building elements which are common between occupancies or common spaces to the habitable spaces of the household units Construction details and/or system specifications. | ☐ | ☐ | ☐ |  |
| ☐ N/A | G7 Natural light | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | General: scope (AS1, AS2, VM1) (applies to housing, old people’s homes and early childhood centres) | ☐ | ☐ | ☐ |  |
|  | Illuminance Awareness of the Outside Environment | ☐ | ☐ | ☐ |  |
| ☐ N/A | G8 Artificial Light | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Artificial lighting locations: all exitways, access routes and common spaces (see G8.2 for limitations) Illuminance: minimum 20 lux min at floor level or NZS 6703 Section 11 | ☐ | ☐ | ☐ |  |
| ☐ N/A | G9 Electricity | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Accessible light switches and plug sockets: details and locations provided for buildings intended for use by persons with disabilities and accessible accommodation | ☐ | ☐ | ☐ |  |
|  | Emergency Power systems: details provided where required for an essential service, listed on Compliance Schedule application | ☐ | ☐ | ☐ |  |
| ☐ N/A | G10 – G11 Piped Services & Gas as an Energy Source | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Specifications provided: pipework construction, installation etc. Flues: location, materials, safety devices, fire dampers, Automatic Extinguishers, Protection of Supply and Gas Meter location | ☐ | ☐ | ☐ |  |
| ☐ N/A | G12 Water supplies | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Alteration to an existing building or Change of use assessment provided (where Section 118 applies) | ☐ | ☐ | ☐ |  |
|  | Supply type: town, rainwater, bore, other water supply (potable or non-potable) If private supply, documentation provided to show: - Description of water source, scheme and operation - Copy of any ORC consents/test results - Testing, maintenance and operational requirements - Treatment systems | ☐ | ☐ | ☐ |  |
|  | Protection of Potable Water: hazard and backflow protection information | ☐ | ☐ | ☐ |  |
|  | Non-potable Supply: protection, outlet and pipeline identification | ☐ | ☐ | ☐ |  |
|  | Water supply: access, support, overflow, HWC / header if tank in roof space | ☐ | ☐ | ☐ |  |
|  | Hot water supply system: type (electric, gas, solar), size, schematic, seismic restraint, safe tray, wetback Safe water temperature: early childhood centres, schools, old people’s homes, institutions for people with psychiatric or physical disabilities, hospitals (45 degrees). All other buildings (50 degrees) | ☐ | ☐ | ☐ |  |
| ☐ N/A | G13 Foul Water: Plumbing | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Waste length to floor waste gully Floor waste gullies are charged by fixture in same room or tundish | ☐ | ☐ | ☐ |  |
|  | Waste pipe diameter, length, material and gradients | ☐ | ☐ | ☐ |  |
| ☐ N/A | G13 Foul Water: Stack System | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Schematic provided for any stack system | ☐ | ☐ | ☐ |  |
|  | Stacks, vents & branches: size, gradients, lengths provided, exclusion zones shown | ☐ | ☐ | ☐ |  |
|  | Mid floor joists/beam layout allow passage of stack/branches | ☐ | ☐ | ☐ |  |
| ☐ N/A | G13 Foul Water: Suspended and subsoil drainage | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Foul water type: Council reticulation, private reticulation, onsite disposal etc Reticulated services: connection to public sewer shown on plans and aligns with QLDC mapping or as-built plans (refer GIS mapping) Onsite waste water system: application complete, report, design and calculations provided Onsite Wastewater Disposal Guidance ☐ AF OSW Application Form completed | ☐ | ☐ | ☐ |  |
|  | Drainage plan showing: main drain, branches, gradients & lengths provided Venting details (terminal vent and unvented branch drains) Access points (i.e. inspection points, rodding points etc.) Overflow relief gullies (ORG)/Gully traps: location and charged | ☐ | ☐ | ☐ |  |
|  | Grease traps: details and size | ☐ | ☐ | ☐ |  |
|  | Angle of influence, depth and proximity of drain to building | ☐ | ☐ | ☐ |  |
|  | Plumbing pipes not running through point load pad or slab thickenings (cross-check against truss plan or engineering’s drawings) | ☐ | ☐ | ☐ |  |
| ☐ N/A | G14 Industrial Liquid Waste | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Security details provided | ☐ | ☐ | ☐ |  |
|  | Treatment and disposal: NUO requirements, discharge with or without pre-treatment to comply with G13/AS2 or G14/VM1 | ☐ | ☐ | ☐ |  |
| ☐ N/A | G15 Solid Waste | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Capacity of Containers and Storage areas: 80 litres per dwelling (multi-unit and group dwellings), space, ventilation provided | ☐ | ☐ | ☐ |  |
|  | Carrying distance: multi-unit and group dwellings, 30 m from chute or storage area | ☐ | ☐ | ☐ |  |
|  | Solid Waste Storage Areas: opening windows, water supply, ventilation, mechanical ventilation, access, another acceptable solution NZS 4303 | ☐ | ☐ | ☐ |  |
|  | Solid Waste Chutes: details provided | ☐ | ☐ | ☐ |  |

| # H1 ENERGY EFFICIENCY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ☐ N/A | H1 Energy Efficiency | ☐ N/A |  |  | COUNCIL USE ONLY |
|  |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| Reference pages | Building Thermal envelope: QLDC – Climate zone 6 Method demonstrates compliance and building within scope of chosen method: - Schedule method - Calculation method - Modelling method (VM1/VM2) Supporting documentation provided including construction R values for each element (not insulation) (Part 2) BRANZ H1 Energy Efficiency Support guidance | ☐ | ☐ | ☐ |  |
|  | Energy efficiency of HVAC systems in commercial buildings (VM3) | ☐ | ☐ | ☐ |  |
|  | Artificial lighting: commercial and communal non-residential buildings (over 300 m2) | ☐ | ☐ | ☐ |  |

# Draft Compliance Schedule

Specified systems

☐ N/A skip to next section - construction documentation ☐ New Compliance Schedule required

☐ Existing Compliance Schedule - specified systems being altered, added, or removed in the course of the building work Existing compliance schedule number: Click here to enter text. For all new or altered specified systems, complete the associated orange sections below for performance standards and inspection/maintenance procedures Please note: Some systems list the most common standard as an option. If using a standard not listed below, please select ‘other’ and complete a specified system form for that system. For further guidance on specified systems, please refer to the MBIE Compliance Schedule Handbook

| SS1 - Auto Systems for Fire Suppression |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with fire report. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4541:2020 Automatic fire sprinkler systems (original version May 2020), Parts 1 - 11 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4541:2020 Automatic fire sprinkler systems (original version May 2020), Part 12 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS2 - Automatic or manual emergency warning systems |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with fire report. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4512:2021 Fire detection and alarm systems in buildings (original version May 2021), Parts 1-5 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4512:2021 Fire detection and alarm systems in buildings (original version May 2021), Part 6 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS3/1 – Automatic Doors |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with specifications. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4239:1993 Automatic sliding door assemblies (original version December 1993) | ☐ |  |  |  |  |
| AS 5007:2007 Powered doors for pedestrian access & egress (original version September 2007), Section 5 | ☐ |  |  |  |  |
| AS 4290:2000 Design and installation of revolving doors (original version January 2000) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook: 2014 Amendment 3, dated 14 February 2014 (Clause 3.1 B.5-B.7 & B.8-B.26, C.1-C.5) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS3/2 - Access controlled doors |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with specifications. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| Association of Building Compliance - Code of Practice for Electro-mechanical controlled locking devices on egress doors:2018 (Version 2), Part 5 and 6 | ☐ |  |  |  |  |
| C/AS2 Acceptable Solution for Buildings other than Risk Group SH (First Edition, Amendment 3, dated 2 November 2023), Clause 3.15.7 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Association of Building Compliance - Code of Practice for Electro-mechanical controlled locking devices on egress doors:2018 (Version 2), Part 7 | ☐ |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014) Clause 3.2 B.2-B.4 & B.5-B.7 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS3/3 - Interfaced fire or smoke doors or windows |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with fire report and specifications. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| C/AS2 Acceptable Solution for Buildings other than Risk Group SH (First Edition, Amendment 3, dated 2 November 2023), paragraphs 3.15.9 and 3.15.10 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 3.3 B.3-B.5 & B.6-B.8 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS4 - Emergency lighting systems |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with EML PS1 and fire report. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| AS 2293.1:2005 Emergency escape lighting & exit signs for buildings, Part 1: System design, installation & operation (Amendment 2, August 2014), & AS 2293.3:2005, Part 3: Emergency escape luminaries and exit signs (Amendment 2, December 2012), as modified by Appendix B of F6/AS1 (Third edition, Amendment 4, dated Jan 2017) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| AS/NZS 2293.2:1995 Emergency escape lighting & exit signs for buildings Part 2: Inspection & maintenance (Amendment 3, December 2012) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS5 - Escape route pressurisation systems |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance, Inspection and Maintenance Standards and Procedures |  | ☐ | ☐ | ☐ | Specified System form reviewed for compliance. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| Please complete a specified system form | ☐ |  |  |  |  |

| SS6 - Riser Mains |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with fire report. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4510:2008 Fire hydrant systems for buildings, incorporating (Amendment 1, dated 30 January 2009), Parts 1-8 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4510:2008 Fire hydrant systems for buildings, (Amendment 1, dated 30 January 2009), Part 9 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS7 - Automatic Backflow Prevention Devices |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with product specifications. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| G12/AS1 Water Supplies Acceptable Solution (3rd Edition Amendment 14, dated 2 November 2024), Clause 3.7.1- 3.7.4 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| AS/NZS 2845.3:2020 Water supply – Backflow prevention devices, Part 3: Field testing and maintenance of testable devices (Original Version, February 2020) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS8.1 - Passenger-carrying lifts |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with product specifications. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| EN 81-20:2014 Safety rules for the construction and installation of lifts – Lifts for the transport of persons and goods (Amendment 1, dated 30 November 2015) – as modified by D2/AS1 (Second Edition, Amendment 7, dated Jan 2017) - Part 20: Passenger and goods passenger lifts | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 8.1 (for EN 81 (Parts 1 & 2) – Refer to checklist) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS8.2 – Platform, low-speed and service lifts |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with product specifications. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4334:2012 Platform lifts and low-speed lifts (original version July 2012), Parts 1-11 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4334:2012 Platform lifts and low-speed lifts (original version July 2012), Appendix A | ☐ |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 8.2 (Refer to checklist) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS8.3 – Escalators and moving walks |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with product specifications. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| EN 115:2008 Safety of escalators and moving walks, Part 1 Construction and installation (Amendment 1 May 2010) as modified by D2/AS3 (Second Edition, Amendment 7, dated Jan 2017) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| EN 115:2008 Safety of escalators and moving walks, Part 1 Construction and installation (Amendment 1 May 2010), Clause 7.4 | ☐ |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 8.3 (Refer to checklist) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS9 – Mechanical ventilation or air conditioning systems |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with HVAC PS1 and specification. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| AS 1668.2:2002 The use of ventilation and air-conditioning in buildings – Part 2: Ventilation design for indoor air contaminant control (Fourth edition, Amendment 2, dated June 2003) | ☐ |  |  |  |  |
| NZS 4303:1990 Ventilation for acceptable indoor air quality (original version September 1990) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| AS/NZS 3666.2:2011 Air-handling and water systems of buildings – Microbial control – Part 2: Operation and maintenance (Third edition, dated 14 November 2011) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS10 - Building maintenance units |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard, Inspection & Maintenance Procedures |  | ☐ | ☐ | ☐ | Specified System form reviewed for compliance. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| Please complete a specified system form | ☐ |  |  |  |  |

| SS11 - Laboratory fume cupboards |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance, Inspection and Maintenance Standards and Procedures |  | ☐ | ☐ | ☐ | Specified System form reviewed for compliance. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| Please complete a specified system form | ☐ |  |  |  |  |

| SS12.1 - Audio loops |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with specifications. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4121:2001 Design for access and mobility - Buildings and associated facilities (original version April 2001) - Appendix H | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4121:2001 Design for access and mobility - Buildings and associated facilities (original version April 2001) - Appendix H | ☐ |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 12.1 B.1-B.3 |  |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS12.2 - FM radio frequency systems and infrared beam transmission systems |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with specifications. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4121:2001 Design for access and mobility - Buildings and associated facilities (original version April 2001) - Appendix H | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4121:2001 Design for access and mobility - Buildings and associated facilities (original version April 2001) - Appendix H |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 12.2 B.1-B.6 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS13 Smoke Control Systems (SS13/1 Mechanical, SS13/2 Natural & 13/3 Curtain) |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance, Inspection and Maintenance Standards and Procedures |  | ☐ | ☐ | ☐ | Specified System form reviewed for compliance. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| Please complete a specified system form | ☐ |  |  |  |  |

| SS14.1 - Emergency power systems for specified systems 1-13 |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with electrical documentation and fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 6104:1981 Specification for emergency electrical supply in buildings (First edition, dated May 1981) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 6104:1981 Specification for emergency electrical supply in buildings (First edition, dated May 1981), Part 8 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS14.2 - Signs relating to specified systems 1-13 |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with specifications and fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| F8/AS1 Signs Acceptable Solution (2nd Edition Amendment 4, dated 1 January 2017) Section 5 Fire related safety features | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 14.2 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS15.1 - Systems for communicating spoken information intended to facilitate evacuation |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with the fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| NZS 4512:2021 Fire detection and alarm systems in buildings (original version May 2021) | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| NZS 4512:2021 Fire detection and alarm systems in buildings (original version May 2021), Section 6 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS15.2 - Final exits |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with the fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| C/AS2 Acceptable Solution for Buildings other than Risk Group SH (First Edition, Amendment 3, dated 2 November 2023), Part 3: Means of Escape | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 15.2 B.3-B.8 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS15.3 – Fire separations |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with the fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| C/AS2 Acceptable Solution for Buildings other than Risk Group SH (First Edition, Amendment 3, dated 2 November 2023), Part 4: Control of internal fire and smoke spread | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 15.3 B.4-B.22 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS15.4 – Signs for communicating information intended to facilitate evacuation |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns with the fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| F8/AS1 Signs Acceptable Solution (2nd Edition Amendment 4, dated 1 January 2017), Section 4 Exit Signs | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| AS/NZS 2293.2:1995 Emergency escape lighting & exit signs for buildings, Part 2: Inspection & maintenance (Amendment 3, December 2012) | ☐ |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 15.4 B.1-B.8 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS15.5 – Smoke separations |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance Standard |  | ☐ | ☐ | ☐ | Check applicants selected means of compliance aligns the fire report. Add performance, inspection, and maintenance standards added to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| C/AS2 Acceptable Solution for Buildings other than Risk Group SH (First edition, Amendment 3, dated 2 November 2023), Part 4: Control of internal fire and smoke spread | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |
| Inspection/Maintenance Procedure |  |  |  |  |  |
| Compliance Schedule Handbook:2014 (Amendment 3, dated 14 February 2014), Clause 15.5 B.3-B.17 | ☐ |  |  |  |  |
| Other (complete a specified system form) | ☐ |  |  |  |  |

| SS16 - Cable Cars |  | Yes | N/A | RFI | Reasons for Decisions/ Comments |
| --- | --- | --- | --- | --- | --- |
| Performance, Inspection and Maintenance Standards and Procedures |  | ☐ | ☐ | ☐ | Specified System form reviewed for compliance. Add performance, inspection, and maintenance standards to draft compliance schedule. Performance Standard: Inspection/Maintenance Procedure: |
| Please complete a specified system form | ☐ |  |  |  |  |

| COUNCIL USE ONLY |  | Yes | N/A | RFI |  |
| --- | --- | --- | --- | --- | --- |
| Draft compliance schedule prepared for issuing with Building Consent Move to issued folder when granting building consent |  | ☐ | ☐ | ☐ | Draft compliance schedule finalised - performance, inspection and maintenance standards align with decisions recorded above. Move draft to issued folder. |

| # Construction Documentation & Producer Statements See IS 25 BC Conditions Construction Documentation and Advice Notes for guidance |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Select as applicable | PS4 Producer Statement – Construction Review | IS 25 # | Yes | N/A | RFI | COUNCIL USE ONLY |
| ☐ | Fire Protection | 37 | ☐ | ☐ | ☐ |  |
| ☐ | Structural Engineer | 26 | ☐ | ☐ | ☐ | If a PS4 is required, snip and paste a copy of the construction monitoring schedule here. |
| ☐ | Façade Engineer | 10 | ☐ | ☐ | ☐ |  |
| ☐ | Mechanical Ventilation | 57 | ☐ | ☐ | ☐ |  |
| ☐ | Other Click or tap here to enter text. |  | ☐ | ☐ | ☐ | Full wording of producer statement requirements to be written exactly how it should appear on Form 5 |
| Select as applicable | PS3 Producer Statement – Construction | IS 25 # | Yes | N/A | RFI | COUNCIL USE ONLY |
| ☐ | Membrane (Shower) | 6 | ☐ | ☐ | ☐ |  |
| ☐ | Intumescent coating | 13 | ☐ | ☐ | ☐ |  |
| ☐ | Membrane (Retaining Wall) | 14 | ☐ | ☐ | ☐ |  |
| ☐ | Plaster Cladding | 15 | ☐ | ☐ | ☐ |  |
| ☐ | Membrane (Roof / Deck) | 16 | ☐ | ☐ | ☐ |  |
| ☐ | Onsite Waste System | 17 | ☐ | ☐ | ☐ |  |
| ☐ | Mechanical Ventilation | 18 | ☐ | ☐ | ☐ |  |
| ☐ | Fire Alarm (PS3/Installers Declaration) | 19 | ☐ | ☐ | ☐ |  |
| ☐ | Sprinkler System (PS3/Installers Declaration) | 22 | ☐ | ☐ | ☐ |  |
| ☐ | Principle Contractor | 25 | ☐ | ☐ | ☐ |  |
| ☐ | Listening System for the Hearing Impaired | 28 | ☐ | ☐ | ☐ |  |
| ☐ | Lift/People Mover | 29 | ☐ | ☐ | ☐ |  |
| ☐ | Glass Barriers | 30 | ☐ | ☐ | ☐ |  |
| ☐ | Emergency Lighting | 34 | ☐ | ☐ | ☐ |  |
| ☐ | Passive Fire Protection | 36 | ☐ | ☐ | ☐ |  |
| ☐ | Electromagnetic or Automatic Doors or Windows | 54 | ☐ | ☐ | ☐ |  |
| ☐ | Automatic Backflow Preventer | 56 | ☐ | ☐ | ☐ |  |
| ☐ | Smoke Control Systems | 58 | ☐ | ☐ | ☐ |  |
| ☐ | Other Click or tap here to enter text. |  | ☐ | ☐ | ☐ | Full wording of producer statement requirements to be written exactly how it should appear on Form 5 |
| Select as applicable | Certification | IS 25 # | Yes | N/A | RFI | COUNCIL USE ONLY |
| ☐ | Setout, Floor Level – Surveyor Certification | 4 | ☐ | ☐ | ☐ |  |
| ☐ | Compaction Certification (fill over 600 mm) | 8 | ☐ | ☐ | ☐ |  |
| ☐ | Geotechnical Certificate (Ground bearing) | 9 | ☐ | ☐ | ☐ |  |
| ☐ | Fire Alarm Certificate | 20 | ☐ | ☐ | ☐ |  |
| ☐ | Sprinkler System Certificate | 21 | ☐ | ☐ | ☐ |  |
| ☐ | Fire Doors | 23 | ☐ | ☐ | ☐ |  |
| ☐ | Acoustic | 27 | ☐ | ☐ | ☐ |  |
| ☐ | Setout (Siting of building) Surveyor Certification | 32 | ☐ | ☐ | ☐ |  |
| ☐ | Potable Water | 46 | ☐ | ☐ | ☐ |  |
| ☐ | Manufacturers Certificate (Design and Manufacture) | 52 | ☐ | ☐ | ☐ |  |
| ☐ | Manufacturers Certificate (Manufacture only) | 53 | ☐ | ☐ | ☐ |  |
| ☐ | Riser Mains Certificate | 55 | ☐ | ☐ | ☐ |  |
| ☐ | Other Click or tap here to enter text. |  | ☐ | ☐ | ☐ | Full wording of certification requirements to be written exactly how it should appear on Form 5 |
| Truss Design/ Fabrication |  | 1 | ☐ | ☐ |  |  |
| Gasworks Certification |  | 2 | ☐ | ☐ |  |  |
| Electrical Certification |  | 39 | ☐ | ☐ |  |  |

| COUNCIL USE ONLY |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | IS 25 # | Yes | N/A | RFI | COUNCIL USE ONLY |
| Section 67 – Waiver or modification | 5 | ☐ | ☐ | ☐ | State what code clause you are waiving/modifying​ ​​State any conditions if required |
| Section 73 – Building on land subject to natural hazards | 7 | ☐ | ☐ | ☐ | State which hazard: erosion, falling debris, subsidence, inundation, slippage ​ |
| Section 75 – Construction of building on 2 or more allotments | 42 | ☐ | ☐ | ☐ |  |
| Section 113 – Buildings with specified intended lives | 35 | ☐ | ☐ | ☐ | Specify intended life ​ ​ ☐ Update TechOne Custom Fields |
| Advice Notes | IS 25 # | Yes | N/A | RFI | COUNCIL USE ONLY |
| Services Demolition | 3 | ☐ | ☐ | ☐ |  |
| Future Proofing | 12 | ☐ | ☐ | ☐ |  |
| Domestic Heating appliance | 33 | ☐ | ☐ | ☐ |  |
| Heritage New Zealand | 38 | ☐ | ☐ | ☐ |  |
| Certificate of Public Use | 41 | ☐ | ☐ | ☐ |  |
| New relocatable building | 45 | ☐ | ☐ | ☐ |  |
| 3rd Party Verification | 47 | ☐ | ☐ | ☐ |  |
| Staged Consent Where building work relies on an earlier stage to achieve compliance, and CCC has not been issued for that stage. | 49 | ☐ | ☐ | ☐ | Click here to enter previous stage BC number for advice note |
| Specified System Plan | 50 | ☐ | ☐ | ☐ |  |
| Other | 51 | ☐ | ☐ | ☐ | Full wording of advice note to be written exactly how it should appear on Form 5 |

| Inspections See IS SI Site Inspection Descriptions for guidance |  |  |
| --- | --- | --- |
| ☐ Pre Construction Meeting | ☐ Framing and Substrates | ☐ Tile Membrane |
| ☐ Foundation and Slab | ☐ Cladding | ☐ Pool and Spa Barrier |
| ☐ Drainage | ☐ Half High Veneer | ☐ Specified Systems |
| ☐ Concrete and Blockwork | ☐ Pre Lining | ☐ Final |
| ☐ Tanking | ☐ Post Lining |  |
| Compliance Schedule |  |  |
| Choose an item. |  |  |
| Existing compliance schedule number: Click here to enter text. |  |  |

| # COUNCIL USE ONLY- FINAL SIGN OFF AND TECH ONE CHECK |  |  |
| --- | --- | --- |
| BC NUMBER Click here to enter text. |  |  |
| Building Category | ☐ Correct. If changed, state why: Click here to enter text. |  |
| Classified Use | ☐ Correct. If changed, state why: Click here to enter text. |  |
| Description of Work | ☐ Correct. If changed, state why: Click here to enter text. |  |

| APRROVED | ☐ I am satisfied ‘on reasonable grounds’ that the Building Consent Documentation to be stamped ‘Approved’ demonstrates compliance with the Building Code, and the Building Consent/Amendment is approved and ready for granting, as per Section 49 of the Building Act 2004 |  |
| --- | --- | --- |
|  | Processing Officer Name: Click here to enter text. | Date: Click here to enter text. |
| REFUSED | ☐ I am NOT satisfied that the Building Consent Application Documentation received demonstrates compliance with the Building Code, and the Building Consent/Amendment is therefore recommended for Refusal under Section 50 of the Building Act 2004 |  |
|  | Processing Officer Name: Click here to enter text. | Date: Click here to enter text. |
| Supervision Sign Off (if required) |  |  |
| Supervisor Name: Click here to enter text. |  | Date: Click here to enter text. |
| Comments Click here to enter text. |  |  |